FEULING® TWIN CAM® ENGINE TECH MEMO

The TC-88 and TC-88B engines have a small, die cast, crank-speed gerotor style oil pump that utilizes a spring washer to preload the pump gears against the cam support plate and pump housing. The side loading from the spring washer can cause excessive friction and premature wear.

The TC-88 engine uses a dry-sump lubrication system. To perform properly, the dry sump system requires a properly sized, BALANCED SYSTEM including a scavenge pump that is capable of removing all residual oil from the crankcase at all engine speeds and conditions. Failure of the scavenge pump to perform to these requirements causes “WET SUMPING.” Instead of returning to the oil tank, the oil builds up in the crankcase and cam chest causing excessive friction/power loss and oil aeration.  This can result in oil being forced out the breather, air cleaner contamination, oil leaks, loss of oil supply, lifter clatter, component wear and potential engine failure.

TC-88 engines have piston cooling jets that are designed to squirt oil on the under side of the pistons. These jets have a check valve that opens at approximately 25-28 PSI oil pressure.  Without adequate pressure pump volume this pressure regulated oil flow to the jets can short circuit critical oil flow from the lifters and top end.

Twin Cam valve springs need oil for cooling.  Inadequate oil flow can cause excessive valve spring temperature causing loss of spring tension resulting in noisy valve train separation and potential engine failure. Inadequate oil supply to the rocker boxes can cause excessive rocker arm bushing wear and component failure.

The Twin Cam® crankshaft has machined “paddles” on the periphery of the left flywheel to provide a trigger for the crank position sensor (CPS).  These paddles can whip up un-scavenged oil in the crankcase, turning the oil into aerated frothing “peanut butter” that pushes its way up into the rocker boxes-out the breather, into the air cleaner and beyond!

Feuling® Oil Pump Corporation has high efficiency, billet, bolt-in replacement oil pumps for T/C® engines that provide increased supply and scavenge volume over the stock oil pump. Large diameter gerotors, aerospace materials and tolerances create high efficiency pumps without the need for a high friction spring washer. The Feuling® Lifters, camplates, and pushrods are designed to optimize the Feuling® oil pumps creating a BALANCED SYSTEM resulting in cooler engine and oil temperatures, more HP and TQ. to the rear wheels.

Feuling’s® oil filter cooler is the ultimate in oil cooling and filtration technology. Combining an oil cooler and oil filter in one, used in conjunction with the stock spin on oil filter. Creating cleaner cooler oil!

The dyno-developed and track tested combination of Feuling® components will allow the Twin Cam® engine to provide maximum power, reliability and longevity.

The Feuling® products are THE solution!

LOW OIL PRESSURE 
1. Pressure gauge not reading properly – double check with another mechanical PSI gauge
2. Pressure relief valve in the Camplate is leaking, sticking open, or not seating properly
3. Pinched Oil Pump O-ring(s)
4. Camplate scored at oil pump mounting surface, pressure gears and or housing scored from debris running through oil pump
5. Camplate top plug not sealing, camplate leaking at oil passages – warped camplate
6. Excessive crankshaft pinion shaft run out causing oil cavitation
7. Intermittent oil psi loss on bagger/FL models during hard acceleration - oil tank baffle shifted & blocking oil pick up hole
8. Miss alignment of oil pump – center Camplate to pinion shaft then oil pump to pinion shaft by rotating engine over while tightening bolts
9. Leaky piston cooling jets
10. Leaky Camplate hydraulic tensioners (’07-’11 Including ’06 Dyna models)

NO OIL PRESSURE 
1. Pressure relief valve stuck open in Camplate 
2. Missing plug in Camplate face (boss vs. no boss)
3. Air lock in oiling system – fill oil filter full of oil - If this is the initial engine start up
4. Broken oil pump gears - gears & housings not lubricated properly during install, debris running through engine & or excessive run out-  in pinion shaft
5. Broken piston cooling jets
6. Stock oil pump installed on Feuling Camplate

WET SUMPING / BLOW - BY 
1. To verify if engine is wet sumping, run engine to operating temperature then shut down and pull the allen head plug from bottom of- engine case.  Measure oil and if more than 5oz. drains from the sump then there is a wet sumping issue.
2. Oil level too high - run oil level no more than 3/4 - 7/8 full cold
3. Sub seal of oil pump miss aligned with the scavenge port hole of the engine case.
4. Piston rings not seated, causing oiling system cavitations, run a cylinder leak down test - Vent oil tank - Feuling’s oil tank breather kits (also Piston Cooling Jet stuck open)
5. Scavenge gears and or housing scored from debris running through oil pump causing out of tolerance scavenge side of oil pump
6. Excessive run out in pinion shaft - causing oil pump / system cavitation
7. Pinched Oil Pump O-ring(s)
8. FL/Baggers – 6 speed trans with dipstick in rear of trans can allow more oil to be added to oil tank and not show on dipstick
9. Excessive RPM on rev limiter, de-seated rings
10. Flapper (Umbrella) valves in breathers stuck
11. Return oil passages plugged, pick-up hole in crankcase plugged

DIPSTICK BLOW OUT 
1. Excessive cylinder leak down – run a cylinder leak down test
2. Oil level too high, run oil level 3/4 - 7/8 full cold
3. Rocker housing gaskets are wrong or installed improperly
4. Spark knock, piston detonation, rings not seated or coming un-seated under hard acceleration
5. Excessive RPM on rev limiter, de-seating rings
6. Vent oil tank – see Feuling oil tank breather kits

NOISY VALVE-TRAIN 
1. Lifter to lifter bore clearance too large, Feuling recommends a clearance of 0.001” - 0.0015” for proper lifter performance
2. Pushrods flexing and hitting pushrod tubes - look for a shiny ring witness mark around pushrod normally seen towards the cylinder head. 
3. Low oil pressure
4. Rocker arms/bushings out of tolerance
5. Valve spring to camshaft combination creating valve-train separation – See Feuling Beehive valve springs
6. Leaky/broken piston cooling jets
7. Clearance for roller rocker arms on under side of rocker box cover
8. Steep ramped camshafts, valves closing so fast the valves bounce off valve seats 
9. Pushrod center oil hole plugged